1、System topology

Connection between electrical node and equipment

| Electrical Node | Berths | QCs | BSS | Wind Turbine (WT) Units |
| --- | --- | --- | --- | --- |
| 2 | — | — | 1 |  |
| 3 | — | — | — | WT1 (5MW) |
| 4 | 4 | 7,8 | — | — |
| 5 | 3 | 5,6 |  | — |
| 6 | — | — | — | WT2 (5MW) |
| 8 | 2 | 3,4 | — | — |
| 9 | 1 | 1,2 | — | — |
| 11 | 5 | 9,10 | — | WT3 (5MW) |
| 12 | 6 | 11,12 | — | — |

2、System data

| Property | Value |
| --- | --- |
| Number of available berths | 6 |
| Number of AESs | 8 |
| Number of available QCs | 12 |
| Number of batteries | 30 |
| Number of AGVs | 20 |
| Number of WT units | 3 |

| AES Parameters |  |  |
| --- | --- | --- |
| Property | Symbol | Value |
| AES arrival time (hour) |  | [1, 2, 4, 5, 6, 9, 10, 13] |
| AES latest departure time (hour) |  | [11, 14, 15, 17, 19, 23, 20, 24] |
| AES service load (MW) |  | [3.0, 3.5, 3.0, 2.5, 4.0, 3.5, 2.5, 4.0] |
| Maximum/Minimum power of AES auxiliary generator (MW) |  | Maximum [6, 7, 6, 5, 8, 7, 5, 8] Minimum [0.6, 0.7, 0.6, 0.5, 0.8, 0.7, 0.5, 0.8] |
| Generation coefficients of AES auxiliary generator ($/MW2, $/MW, $) | , , | [6.615,6.615,6.615,7.497,7.056,7.056,6.615,6.615] [85.995,83.349,79.380,84.672,85.995,92.610,92.610,89.964] [58.810,66.150,69.825,66.150,69.825,51.450,55.125,73.510] |
| Maximum charging/discharging power of ESS on AES (MW) |  | Charging [4.50, 5.25, 4.50, 3.75, 6.00, 5.25, 3.75, 6.00] Discharging [4.50, 5.25, 4.50, 3.75, 6.00, 5.25, 3.75, 6.00] |
| Minimum charging/discharging power of ESS on AES (MW) |  | Charging [0, 0, 0, 0, 0, 0, 0, 0] Discharging [0, 0, 0, 0, 0, 0, 0, 0] |
| Maximum/minimum energy level of ESS on AES (MWh) |  | Maximum [8.10, 9.45, 8.10, 6.75, 10.80, 9.45, 6.75, 10.80] Minimum [0.90, 1.05, 0.90, 0.75, 1.20, 1.05, 0.75, 1.20] |
| Maximum power of G2B and B2G (MW) |  | G2B [30, 30, 30, 30, 30, 30, 30, 30] B2G [30, 30, 30, 30, 30, 30, 30, 30] |
| Minimum power of G2B and B2G (MW) |  | G2B [0, 0, 0, 0, 0, 0, 0, 0] B2G [0, 0, 0, 0, 0, 0, 0, 0] |
| Initial energy level of ESS on AES (MWh) |  | [0.90, 1.05, 0.90, 0.75, 1.20, 1.05, 0.75, 1.20] |
| Charging/discharging efficiency of ESS on AES |  | Charging [0.95, 0.95, 0.95, 0.95, 0.95, 0.95, 0.95, 0.95] Discharging [0.95, 0.95, 0.95, 0.95, 0.95, 0.95, 0.95, 0.95] |
| Maximum/minimum number of QCs that can be assigned for AES |  | Maximum [5, 5, 5, 4, 4, 5, 5, 5] Minimum [1, 1, 1, 1, 1, 1, 1, 1] |
| Number of cargoes on AES (TEU) |  | [650, 760, 650, 550, 580, 770, 640, 950] |

| QC Parameters (All QC are considered homogeneous) |  |  |
| --- | --- | --- |
| Property | Symbol | Value |
| Maximum cargo handling efficiency of QC (TEU/hour) |  | 50 |
| Duration of lifting up/down in one cargo handling cycle (hour) | , | Lifting up 0.00833 Lifting down 0.00833 |
| Power demand when lifting up (MW) |  | 1.2 |
| Maximum regenerated power when lifting down (MW) |  | 0.6 |
| Maximum charging/discharging power of ESS on QC (MW) |  | Charging 1 Discharging 1 |
| Minimum charging/discharging power of ESS on QC (MW) |  | Charging 0 Discharging 0 |
| Maximum/minimum energy level of ESS on QC (MWh) |  | Maximum 1.8 Minimum 0.2 |
| Initial energy level of ESS on QC (MWh) |  | 1.8 |
| Charging/discharging efficiency of ESS on QC |  | 0.95 |

| AGV and BSS Parameters (All AGVs and batteries are considered homogeneous) |  |  |
| --- | --- | --- |
| Property | Symbol | Value |
| Maximum transport efficiency of AGV when working/swapping battery (TEU/hour) |  | Working 25 Swapping battery 20 |
| Maximum/minimum energy level of battery (MWh) |  | Maximum 0.1672 Minimum 0.0088 |
| Maximum charging/discharging power of battery (MW) |  | Charging 0.45 Discharging 0.45 |
| Minimum charging/discharging power of battery (MW) |  | Charging 0 Discharging 0 |
| Coefficient to represent the time loss caused by swapping battery |  | 0.8 |
| Initial energy level of battery (MWh) |  | 0.1672 |
| Threshold on energy level of battery for swap-in/out (MWh) | , | 0.1408 0.0352 |
| Charing/discharging efficiency of battery |  | Charging 0.95 Discharging 0.95 |
| Energy consumption of battery on AGV in one cargo transport cycle (MWh) |  | 0.0012672 |

| SPDN Parameters |  |  |
| --- | --- | --- |
| Property | Symbol | Value |
| Constant active and reactive power loads (MW, MVar) | , | All nodes are assumed to have the same loads; Active power load at 24 hours [1.77, 1.63, 1.85, 1.30, 1.46, 1.33, 2.01, 1.94, 2.38, 1.46, 2.46, 2.76, 2.69, 2.71, 2.68, 2.26, 2.07, 2.01, 1.87, 1.70, 1.77, 2.55, 2.30, 2.09] Reactive power load at 24 hours [1.25, 1.34, 1.34, 1.35, 1.36, 1.39, 1.56, 1.65, 1.62, 1.60, 1.87, 1.87, 1.86, 1.85, 1.86, 1.64, 1.48, 1.41, 1.46, 1.35, 1.39, 1.62, 1.62, 1.42] |
| Maximum/minimum voltage magnitude (p.u.) | , | Maximum 1.05 Minimum 0.95 |
| Forecast renewable generation (MW) |  | Every renewable energy unit has the same forecast output; The forecast output of one renewable energy unit at 24 hours [2.20, 3.51, 3.80, 4.10, 4.20, 4.20, 5.00, 5.00, 3.90, 3.20, 5.00, 4.60, 4.20, 4.00, 3.90, 1.60, 0.20, 0.40, 0.50, 0.25, 0.30, 2.80, 4.10, 2.60] |
| Day-ahead electricity purchase price ($/MW) |  | [117.5, 59.0, 43.5, 36.0, 43.0, 131.5, 142.5, 152.0, 164.0, 148.5, 144.5, 137.5, 139.0, 137.0, 136.0, 140.5, 148.5, 147.5, 137.5, 132.0, 126.0, 124.0, 121.5, 33.5] |
| Intra-day electricity adjustment price ($/MWh) | , | Day-ahead [127.5, 126.0, 79.5, 47.5, 64.5, 145.5, 147.5, 171.0, 185.5, 160.5, 170.5, 148.0, 149.5, 152.0, 158.0, 163.5, 160.5, 161.5, 147.0, 147.5, 152.5, 143.0, 142.5, 35.0] Intra-day [127.5, 126.0, 79.5, 47.5, 64.5, 145.5, 147.5, 171.0, 185.5, 160.5, 170.5, 148.0, 149.5, 152.0, 158.0, 163.5, 160.5, 161.5, 147.0, 147.5, 152.5, 143.0, 142.5, 35.0] |